Supplementary information

Supplementary Table S1. Studies comparing LUR models with other exposure models for estimation of NO2

| Study, country, pollutant average | Exposure models compared | Methods of comparison | Comments |
| --- | --- | --- | --- |
| Buteau et al, 2017 Canada; Predictions made for 96 postal codes (Buteau et al. 2017) | NO2 (daily averages) Nearest-monitor vs IDW Nearest monitor vs LUR | Levels of agreement ICC=0.81 ICC=0.60 | Maximum differences in NO2 for given days: 74 ppb 108 ppb |
| Hennig et al, 2016; Germany; Predictions for 4809 cohort addresses. (Hennig et al. 2016) | LUR vs CTM (4809 cohort addresses) LUR vs CTM at NO2 sites (Background & traffic-all sites) | Difference of -7.4 ± 4.9 ug/m3 R2=0.55 | Better R2 when CTM method restricted to local traffic areas only. |
| Wang et al, 2015; The Netherlands; Predictions for 1058 cohort addresses. (Wang et al. 2015) | LUR (ESCAPE) vs DM (NO2) | R2=0.47-0.85 |  |
| De Hoog et al, 2014; Predictions for 13 ESCAPE cohort addresses (total n=112971). (de Hoogh et al. 2014) | LUR vs DM | R2=0.19-0.89 B-A plots, no ICCs reported | Pearson R2 varied substantially between the 13 LURs from 13 different countries |
| Sellier et al, 2014; France; Predictions for 776 addresses. (Sellier et al. 2014) | LUR vs AQMS LUR vs TAG LUR vs DM | R2=0.46-0.76 R2=0.73-0.87 R2=0.77-0.87 | Difficult to draw overall comparisons as individual R2 were provided by distance from AQMS and type of area |
| Wu et al, 2011; California, US; Predictions for >81,000 addresses. (Wu et al. 2011) | LUR vs DM (CALINE4) (NOx) LUR vs AQMS (NO2) LUR vs AQMS (NOx) LUR vs traffic density (NO2) | R2=0.49 R2=0.57 R2=0.46 R2=0.27 |  |
| Beelen et al, 2010; The Netherlands; Predictions made at N =69 975 grid points (Beelen et al. 2010) | LUR vs DM (NO2) DM vs validation sites (NO2) LUR vs validation sites (NO2) | R2=0.55 R2=0.77 R2=0.47 | Good agreement at mid-range but larger differences at high and low concentrations. Perhaps due to coarse category for “industrial land use” used in LUR |
| Marshall et al, 2008; Canada; Predictions for 56,099 postal codes. (Marshall et al. 2008) | LUR vs IDW (NO2) LUR vs DM (CMAQ) (NO2) LUR vs AQMS (nearest) (NO2) | R2=0.52 R2=0.49 R2=0.54 | LUR produced lowest estimates. Attributed to postcode centroids not located along roads, hence leading to under-estimation. |

AQMS Air Quality Monitoring Station

DM Dispersion model

CTM Chemical transport model

TAG Temporally adjusted geostatistical model

Supplementary Table S2. LUR variables and sources of data

|  | GIS Data source | Variable Description | Buffer size (m) | SYDNEY NAME | ESCAPE NAME | Calc/units |
| --- | --- | --- | --- | --- | --- | --- |
| Land use | Australian Bureau of Statistics (ABS) | Residential - Mesh Block | 100, 300, 500, 700, 1000, 2500, 5000 | Resmb |  | Proportion of land area |
| Land use | ABS | Industrial - Mesh Block | 100, 300, 500, 700, 1000, 2500, 5000 | Indmb |  | Proportion of land area |
| Land use | ABS | Commercial - Mesh Block | 100, 300, 500, 700, 1000, 2500, 5000 | Commmb |  | Proportion of land area |
| Land use | ABS | Parkland - Mesh Block | 100, 300, 500, 700, 1000, 2500, 5000 | Openmb |  | Proportion of land area |
| Land use | ABS | Water - Mesh Block | 100, 300, 500, 700, 1000, 2500, 5000 | Openmb |  | Proportion of land area |
| Land use | ABS | Park/Water/Agric - Mesh Block | 100, 300, 500, 700, 1000, 2500, 5000 | Openmb |  | Proportion of land area |
| Population Density | ABS | Population within buffers | 100, 300, 500, 700, 1000, 2500, 5000 |  |  | Number |
| Dwelling density | ABS | Dwellings within buffers | 100, 300, 500, 700, 1000, 2500, 5000 |  |  | Number |
| Distance to coast | Geoscience Australia | Distance to Coast |  |  |  | m |
| Altitude | Geoscience Australia | Altitude - SRTM 1 arc second derived DEM | NA | Elevation |  | m |
| Traffic | Zenith_plus_local | Traffic intensity on nearest road | N/A | TRAFNEAR | TRAFNEAR | vpd |
| Traffic | Zenith_plus_local | Distance to nearest road | N/A | DISTINVNEAR1, DISTINVNEAR2 | DISTINVNEAR1, DISTINVNEAR2 | m-1, m-2 |
| Traffic | Zenith_plus_local | Product of traffic count on nearest road & inverse of distance to nearest road & distance squared | N/A | INTINVDIST, INTINVDIST2 | INTINVDIST, INTINVDIST2 | vpd/m; vpd/m2 |
| Traffic | Zenith_plus_local | Traffic intensity on nearest major road | N/A | TRAFMAJOR | TRAFMAJOR | vpd |
| Traffic | Zenith_plus_local | Distance to nearest major road | N/A | DISTINVMAJOR1, DISTINVMAJOR2 | DISTINVMAJOR1, DISTINVMAJOR2 | m-1, m-2 |
| Traffic | Zenith_plus_local | Product of traffic intensity on nearest major road & inverse of distance to nearest major road & distance squared | N/A | INTMAJORINVDIST, INTMAJORINVDIST2 | INTMAJORINVDIST, INTMAJORINVDIST2 | vpd/m; vpd/m2 |
| Traffic | Zenith_plus_local | Traffic load of major roads in buffer (sum of (traffic intensity*length of all major road segments)) | 25, 50, 75, 100, 300, 500, 700, 1000 | LOADMAJ | TRAFMAJORLOAD | Sum of (count*length) major roads |
| Traffic | Zenith_plus_local | Traffic load of all roads in buffer (Sum(traffic counts*length) all segments) | vpd | LOAD | TRAFLOAD | vpd |
| Traffic | Zenith_plus_local | Heavy Traffic intensity on nearest road | vpd | HEAVYTRAFNEAR | HEAVYTRAFNEAR | vpd |
| Traffic | Zenith_plus_local | Product of heavy traffic intensity on nearest road & inverse of distance to nearest rd & distance squared | vpd/m, vpd/m2 | HEAVYINTINVDIST, HEAVYINTINVDIST2 | HEAVYINTINVDIST, HEAVYINTINVDIST2 | vpd/m, vpd/m2 |
| Traffic | Zenith_plus_local | Heavy traffic intensity on nearest major road | vpd | HEAVYTRAFMAJOR | HEAVYTRAFMAJOR | vpd |
| Traffic | Zenith_plus_local | Heavy Traffic load of major roads in buffer (sum of (heavy traffic intensity*length of all major road segments)) | 25, 50, 75, 100, 300, 500, 700, 1000 | HEAVYLOADMAJ | HEAVYTRAFMAJORLOAD | vpd/m |
| Traffic | Zenith_plus_local | Heavy Traffic load of all roads in buffer (sum of (heavy traffic intensity*length of all road segments)) | 25, 50, 75, 100, 300, 500, 700, 1000 | HEAVYLOAD | HEAVYTRAFLOAD | vpd/m |
| Traffic | NSW Land & Property Information (LPI) | Weighted Road Density | 25, 50, 75, 100, 300, 500, 700, 1000 |  |  |  |
| Traffic | LPI | Road Length of all roads in buffer | 25, 50, 75, 100, 300, 500, 700, 1000 | ALLROAD | ROADLENGTH | m |
| Traffic | LPI | Road Length of minor roads in buffer | 25, 50, 75, 100, 300, 500, 700, 1000 | MINROAD | Not in ESCAPE | m |
| Traffic | LPI | Road length of major roads in buffer | 25, 50, 75, 100, 300, 500, 700, 1000 | MAJROAD | MAJORROADLENGTH | m |
| Other NO2 Sources | National pollutant inventory (NPI) | Number of Oxides of Nitrogen sources in buffers | 50, 75, 100, 150, 200, 300, 500, 700, 1000, 2500, 5000, 10000 |  | Not in ESCAPE |  |

Supplementary Table S3.  Summary statistics for NO2/NOx passive sampler concentrations (ppb) by site type

| Site type (n) | Mean | CIs | SD | Min | Max | 25th% | 75th% |
| --- | --- | --- | --- | --- | --- | --- | --- |
| NO2 |  |  |  |  |  |  |  |
| Overall (n=46) | 9.0 | 8.1-9.9 | 3.1 | 3.7 | 17.3 | 7.1 | 10.4 |
| Traffic (n=16) | 11.6 | 9.9-13.2 | 3.4 | 6.0 | 17.3 | 8.7 | 13.6 |
| Urban background (n=24) | 7.7 | 6.9-8.4 | 1.8 | 4.8 | 12.4 | 6.0 | 8.4 |
| Urban Background <100 m to main road (n=4) | 8.5 | 7.7-9.3 | 1.2 | 7.1 | 9.8 | 7.7 | 9.4 |
| Regional (n=2) | 5.3 | 2.2-8.4 | 2.2 | 3.7 | 6.9 | 4.5 | 6.1 |
| NOx |  |  |  |  |  |  |  |
| Overall (n=46) | 17.1 | 14.9-19.4 | 7.9 | 6.6 | 43.4 | 12.4 | 18.2 |
| Traffic (n=16) | 24.1 | 19.7-28.5 | 9.0 | 12.4 | 43.4 | 17.0 | 31.2 |
| Urban background (n=24) | 13.2 | 11.7-14.7 | 3.7 | 6.6 | 24.0 | 11.1 | 15.0 |
| Urban Background <100 m to main road (n=4) | 17.0 | 15.8-18.2 | 1.2 | 15.5 | 18.2 | 16.3 | 17.9 |
| Regional (n=2) | 9.4 | 5.6-13.2 | 2.8 | 7.4 | 11.4 | 8.4 | 10.4 |

Supplementary Table S4.  Comparison of measurements by passive samplers vs fixed site monitors

| Period | Passive sampler | Regulatory fixed site monitor | Comments |
| --- | --- | --- | --- |
| Winter (July 2013 period) |  |  | Excluded due to duplicates exceeding 30% variability and because OEH monitored data was missing for 4 days of the period |
| Summer (Dec 2013) | 5.5 | 8.5 |  |
| Autumn (Mar 2014) | 9.2 | 10.4 |  |

Supplementary Figure S1. Scatter plots of a) LUR vs Sat-LUR (blue) and vs BME (red) NO2 estimates; b) Scatter plot matrix

(ppb)